| BRIEF DESCRIPTION OF WORK / ACTIVITY : Site establishment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA Reference No. |  |  |  | RA Submission Date |  |  |  |  |  | COMPANY : |  |  | SD Approval Date |  |
| RA Revision | 00 |  |  | RA Revision Date |  |  |  |  |  | LOCATION : |  |  |  |  |
| RISK ASSESSMENT TEAM |  |  |  |  | TITLE / APPOINTMENT |  |  |  |  | SIGNATURE |  | COMMENTS |  |  |
| Initials & Surname |  |  |  |  | CAT “A” APPROVAL |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CR 8(1) Contractor Construction Manager |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CR 8(7) Contractor Construction Supervisor |  |  |  |  |  |  |  |  |  |
| Craig Burrows |  |  |  |  | CR 8(5) Contractor SHE Officer |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CAT “B” REVIEW – CLIENT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CR5(5)(6)(7) SHE Agent |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Project SHE Manager |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | CAT “C” REVIEW – CLIENT |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | OHS Act 16.2 Client / Project Manager |  |  |  |  |  |  |  |  |  |
| EXISTING CONTROL MEASURES (SUBMIT & ATTACH TO THE RISK ASSESSMENT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Available | Adequate | Changes Required |
| Procedures : (SOP) |  | Yes |  |  | No |  |  | Yes |  | Remarks |  |  |  |  |
|  |  |  |  |  |  | No |  | Yes | No |  |  |  |  |  |
| Training : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Code of Practice : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 1. | Visitors Induction at main security entrance | Poor access control, ignoring pre-site entry requirements | Delivery truck operator not aware of site related hazards, failing to comply resulting in injury or damages | 3 | 6 | 126 | Operator to be inducted as per requirements prior to site entry. Park truck outside premises and proceed to main security gate. Undergo visitor induction and complete visitors permit at security office. Visitors to comply with full site requirements in regards to PPE ( Hardhat, steel-toe cap safety boots, long sleeve trousers, high visibility vests with zip, safety glasses and hearing protection on standby) | 2 | 2 | 16 | Supervisor |  |
| 1.2 | vehicle inspection by Supervisor | No/ poor inspection conducted by supervisor. | Substandard vehicles allowed onto site, causing accident, injuries, damage. | 3 | 6 | 126 | Delivery vehicle operator to report to gate whom will call relevant company. Operator to conduct site induction at gate 1. All delivery vehicles will be inspected by supervisors using Project required format. Only safe declared vehicles will be granted access onto premises. | 2 | 2 | 16 | Supervisor |  |
|  |  | Unsafe positioned vehicle during site approval inspection. | Interfacing with traffic and not parking in designated parking areas | 3 | 4 | 84 | Do not park vehicle in in road. Stop at safe location next to road andSupervisor to confirm safe parked for inspection and all to remain clear of | 2 | 2 | 16 | Supervisor |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
|  |  |  |  |  |  |  | traffic. Only use designated parking areas. |  |  |  |  |  |
| 1.3 | Escort to laydown area by means of site approved vehicle / flagman | Not escorting delivery truck. No route planning | Causing time delays. No turning point. | 2 | 4 | 34 | Supervisor to plan accordingly to ensure escort vehicle is available at all times. C/S to physically perform proper route planning prior to allowing vehicle to access site and communicate planned route with operator and spotters. | 1 | 2 | 2 | Supervisor |  |
|  |  | Driver under influence of alcohol. | Causing accidents on site, driving over/into personnel. | 3 | 8 | 168 | Driver to be informed to ensure to maintain a sober habit. Driver to be tested for alcohol at access gate. No person under the influence of alcohol will be granted access onto premises. | 2 | 2 | 16 | Supervisor |  |
|  |  | Failure to adhere to site traffic and SHE rules. | Causing accidents on site, driving over/into personnel. | 3 | 8 | 168 | Driver to obey all site traffic and SHE rules, wear safety belt, do not use phone while driving, keep to speed limit of and remove keys from ignition when not in cabin. Wear required PPE. Visitors are not allowed to perform any physical work, no | 2 | 2 | 16 | Supervisor/ Delivery driver |  |

| Task Activity : |  | Site establishment |  |  |  |  |  |  |  | RA REF : |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen |  | Initial Risk Score |  |  | Existing & Proposed Control Measures |  | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |  |
|  |  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |  |
|  |  |  |  |  |  |  |  | visitors are allowed on site without an escort. |  |  |  |  |  |  |  |
| 1.4 | Position truck / LDV to offload | Blind spots while reversing into position. | Colliding with, structures or employees |  | 3 | 8 | 168 | Spotter/ Flag man to be positioned at safe distance behind truck visible in operator side and rear-view mirror instructing operator in reverse actions. Truck operator to immediately stop if spotter is not visible. Spotter not to walk backwards while directing. Remove all obstacles and tripping hazards prior to directing truck. No reverse action without spotter and guidance due to restricted and congested space. |  | 2 | 4 | 32 | Supervisor |  |  |
|  |  | More than one person instructing operator. | Causing confusion. Colliding into structures, trees or other employees. |  | 3 | 4 | 84 | Only one spotter to give instructions and to communicate with other employees to maintain 5m distance at all times. |  | 2 | 1 | 2 | Supervisor |  |  |
| 1.5 | Park Truck / LDV | Fail to switch off the engine of the truck and apply the park brake | Unauthorized person uses the truck. The truck moves whilst busy with the offloading process. |  | 4 | 4 | 160 | The driver of the truck must switch off the engine of the truck and apply the park brake. Place stop blocks in front and behind one of the rear wheels. |  | 4 | 2 | 80 | Supervisor |  |  |
| Amendment no. Rev 0 |  |  |  | Approved by: |  |  |  |  | Date: |  |  |  |  |  |  |
| It is the responsibility of person printing this document to ensure it is the newest revision on the system before using or directing anyone to use this document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
|  |  | and isolate. And install stop blocks | Drives over people. Collide with other vehicles. |  |  |  | The driver and his assistant must remain in his truck for the duration of the offloading process, unless offloaded by mobile crane, then operator to be position in safe area. Keys to be removed from ignition when driver is not in cabin. |  |  |  |  |  |
| 2.1 | Perform offloading location inspection and complete DSTI to include all interfacing, obstacles, area and task related hazards and risks. | Poor/No area inspection conducted to identify all area related hazards. | Employees not aware of area and task related hazards causing Injury to workforce or interfacing employees. Possible damage to plant and equipment. | 4 | 8 | 320 | Supervisor to perform work area inspection and conduct DSTI with all area and task related hazards identified. All relevant parties to sign DSTI. Communicate to all involved staff and acknowledge. DSTI to be revised and re-communicated if any changes in planned scope of work, changes in conditions or area. | 2 | 4 | 32 | Supervisor |  |
|  |  | Supervisor not familiarised with DSTI procedure. | Supervisor not aware of all requirements regarding DSTI’s | 2 | 4 | 34 | Relevant Daily document training to be conducted by S/O for supervision to ensure correct use of all documents | 2 | 2 | 16 | Safety Officer |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 2.2 | Prepare laydown area (tripping hazards) | Drive and walk on Uneven surfaces | Delivery truck topples over causing serious damage to equipment and injuries, losing load | 3 | 6 | 126 | Supervisor to ensure truck offloading location is level, stable and safe to commence prior to entry. | 2 | 4 | 32 |  |  |
|  |  |  | Trip and fall over boulders, rocks and materials causing Ankle sprains | 3 | 6 | 84 | Prepare safe walkways and ensure it is used by all employees. Ensure that workers are aware of the tripping hazards and captured on daily DSTI. Ensure that workers are wearing correct footwear. | 2 | 2 | 16 | Supervisor |  |
| 2.3 | Access truck by means of secured ladder | No safe access provided to access truck | Employees slip and fall causing injuries | 3 | 4 | 84 | Supervisor to ensure to provide sufficient size ladder to access truck. Ensure all employees access truck by making use of safe access point only. | 2 | 2 | 16 | Supervisor |  |
|  |  | Substandard ladder utilized for access | Ladder breaking causing people to fall and sustain injuries | 3 | 4 | 84 | Ladder to be inspected, placed on register with findings noted colour code system if found safe for use | 2 | 2 | 16 | Supervisor |  |
|  |  | Unsecured ladder | Ladder topples over or slip causing employees to fall. | 3 | 4 | 84 | Extra person to secure ladder or ladder to be secured to Rollback/Truck trailer prior to access. | 2 | 2 | 16 | Supervisor |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 2.4 | Removing tie- down straps/chains | Incorrect type straps utilized to strap material/ containers (Satan type) | Sudden release of stored energy from tensioner causing serious injuries to employees. | 3 | 6 | 126 | Supervisor to ensure to identify upon pre-site entry inspection. Service providers to be informed to only utilize ratchet type tensioners. | 3 | 2 | 42 | Supervisor |  |
|  |  | No anchor points and restricted movement space on truck. | Employees falling from delivery truck. | 3 | 6 | 126 | Supervisor and S/O to assess and provide anchor points where required (When above 2m) Access ladder to be utilized to remove tie-down chains/straps if and when movement space on truck is restricted. | 3 | 2 | 42 | Supervisor |  |
| 2.5 | Selecting lifting tackle to lift container with truck mounted crane. | Incompetent person selecting equipment. | Lifting tackle failing causing containers to fall and cause damage, injury to personnel | 4 | 6 | 240 | Operator and supervisor to ensure to hook container only on provided anchor / lifting points. Only trained and certified person to sling and hook containers for lifting, and ensuring | 2 | 2 | 16 | Supervisor/ Operator |  |
|  |  |  |  |  |  |  | correct lifting tackle is used. |  |  |  |  |  |
|  |  | Substandard | Lifting tackle failing causing | 3 | 6 | 126 | S/O and Hi-Up operator to ensure load | 2 | 2 | 16 |  |  |
|  |  | equipment | containers to fall and cause damage |  |  |  | test certificates with inspections conducted by competent appointed person is done prior to use of any lifting |  |  |  | Supervisor/ Operator |  |
|  |  |  |  |  |  |  | tackle. All lifting tackle shall be |  |  |  |  |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
|  |  |  |  |  |  |  | inspected and certified, including operator prior to site entry. |  |  |  |  |  |
|  |  | No area access control. | Employees get struck by container / Overhead lifting | 5 | 6 | 600 | Off-loading area to be barricaded with snow netting or lifting flags and relevant signage displayed. Spotters to be positioned to control access. 2 taglines to be used to control the load. | 2 | 2 | 16 | Supervisor |  |
| 2.6 | Attach lifting tackle to container and Hi-up hook. | Incompetent person attaching lifting tackle. Substandard method of hooking / attaching lifting tackle. | Load falling causing damage to equipment. Injuries to workers. Pinch Points and line of fire. | 3 | 6 | 126 | Only competent person to attach lifting tackle to required anchor points. Ensure to utilize secured and provided anchor points only. | 2 | 2 | 16 | Supervisor/ Rigger |  |
|  |  | No safe access provided to attach lifting equipment to container on truck/trailer | Employees falling from/ slipping from container causing serious injuries | 4 | 6 | 240 | Supervisor to ensure to provide ladder to access desired position. Ladder to be secured at all times when utilized. | 2 | 2 | 16 | Supervisor |  |
|  |  | Fail to secure guide ropes prior to lifting of containers. | No control over suspended container, colliding with plant, machinery or employees. | 4 | 6 | 240 | Rigger and C/S to ensure to attach guide rope/s prior to lifting containers as to ensure full control over suspended containers. | 2 | 2 | 16 | C/S |  |

| Task Activity : |  | Site establishment |  |  |  |  |  |  |  | RA REF : |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen |  | Initial Risk Score |  |  | Existing & Proposed Control Measures |  | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |  |
|  |  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |  |
| 2.7 | Lift slew and lower container into position. | Poor visibility around containers. | Container collide with Plant , equipment, employees causing damage or injuries |  | 3 | 6 | 168 | Supervisor to provide spotter/s where required to communicate during offloading as to avoid any collisions. Signalman to be on site to give clear instructions while positioning containers that is out of sight. |  | 2 | 4 | 32 | Supervisor |  |  |
|  |  | Suspended loads | Employees moving in under suspended loads |  | 4 | 6 | 240 | Offloading area to be properly barricaded and only offloading crew to be authorised in demarcated area. No persons to move in under suspended loads |  | 2 | 2 | 16 | Supervisor |  |  |
|  |  | Offloading containers on uneven ground conditions. | Truck with mounted crane topple over causing damage or serious injuries. |  | 3 | 6 | 126 | Pre planning to be conducted to ensure truck with mounted crane establish on stable, compacted and even ground conditions. Outriggers to be fully extended and outrigger pads to be positioned if required. |  | 2 | 2 | 16 | Supervisor/ Operator |  |  |
| 2.8 | Lifting containers to insert blocks to lift container from ground. | Hands or other body parts under suspended load containers while lowering into position. | Serious hand and finger injuries. |  | 3 | 8 | 168 | C/S to ensure employees manually handle container with hands only when 500mm from ground level. Keep |  | 1 | 4 | 4 | C/S |  |  |
| Amendment no. Rev 0 |  |  |  | Approved by: |  |  |  |  | Date: |  |  |  |  |  |  |
| It is the responsibility of person printing this document to ensure it is the newest revision on the system before using or directing anyone to use this document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
|  |  |  |  |  |  |  | hands out from below container while positioning dunnage where required. |  |  |  |  |  |
| 2.9 | Detach lifting tackle from container and hook. | No safe access provided to detaching lifting equipment from container | Employees falling from/ slipping from container causing serious injuries | 3 | 6 | 126 | Supervisor to ensure to provide ladder to access desired position. Ladder to be secured at all times when utilized. | 2 | 2 | 16 | Supervisor |  |
|  |  | Nip and pinch points | Finger and hand injuries | 2 | 4 | 32 | Employee to wear required Hand protection and keep hands clear of nip points. | 2 | 2 | 16 | Rigger/ Supervisor |  |
| 2.10 | Manually offload; Portable toilets, flammable liquid, tables and chairs | Sharp edges | Lacerations to fingers, hands and other body parts. | 2 | 4 | 32 | Employees to wear required Hand protection and keep hands clear of nip points. | 1 | 2 | 2 | Supervisor |  |
|  |  | Heavy objects | Back sprains, objects falling on employees | 2 | 4 | 32 | Employees to assist with heavy objects, lift with legs not with back. Remove all tripping hazards prior to offloading. Do not walk backwards while carrying items. Max – 25kg per person. Maintain proper | 2 | 2 | 16 | Supervisor |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
|  |  |  |  |  |  |  | communication, do not throw material to each other. |  |  |  |  |  |
| 3.0 | Health & Hygiene | Extreme weather conditions | Heatstroke, Heat exhaustion | 3 | 4 | 84 | Supervisor to ensure to provide clean drinking water. Employees to consume at least 500ml each hour to stay hydrated. Ensure to identify all drinking containers. Supervisor to ensure that workers take regular breaks. Shaded area to be provided. | 2 | 2 | 16 | Supervisor |  |
|  |  | Rain, no protection from wet conditions | Wet clothing, hypothermia. | 2 | 4 | 80 | Eating area with roof and sufficient seating to be provided. Employees to move into eating area in rainy conditions. | 2 | 2 | 16 | Supervisor |  |
|  |  | Not washing hands prior to lunch and no hand wash facility provided. | Health hazards, illness | 3 | 6 | 126 | Employees to ensure to wash hands prior to eating and store food in clean area out of direct sunlight and exposure to dust and chemicals. Supervisor to ensure that hand wash facilities are provided. | 2 | 2 | 16 | Supervisor |  |
|  |  | No toilet facilities | Health hazard. | 3 | 4 | 84 | Contractor to ensure that toilet and wash basin facilities are on site and that they are compliant with Facilities | 2 | 2 | 16 | Supervisor |  |

| Task Activity : |  | Site establishment |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
|  |  |  | Environmental impact |  |  |  | regulations and the toilets are clean and serviced regularly. Toilets shall be at the area of work. Water bowser for hand washing shall be made available with biodegradable, environmentally friendly soap to not have an environmental impact |  |  |  |  |  |
| 4.0 | Environmental | Oil, other chemical spillages | Environmental impact | 3 | 6 | 126 | Drip trays to be used when required. Spill kits shall be made available. MSDS on site and personnel trained. Correct Storage of hazardous chemicals. | 2 | 2 | 16 | Supervisor |  |

| Task Activity : |  | Transporting of Workers |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 1. | Transporting of workers | Incompetent Driver Reckless Driving | Injury or Fatality to workers Injury or fatality to public drivers | 5 | 6 | 600 | Drivers shall drive with care, use the correct routs, drive within the speed limit and follow any other site rules | 3 | 2 | 42 |  |  |
|  |  | Fatigue | Damage to property |  |  |  | Drivers shall be competent, responsible medically fit and hold a valid drivers licence with PDP |  |  |  |  |  |
|  |  |  |  |  |  |  | Managing the working hours of the drivers in order to combat fatigue |  |  |  |  |  |
|  |  |  |  |  |  |  | No overloading of passengers and no transporting passengers and material or equipment in the same cab. |  |  |  | Supervisor |  |
|  |  |  |  |  |  |  | No persons are allowed to be transported on the back of vehicle. All passengers shall have a seat that has been designed for that purpose and all passengers shall have safety belts and shall be used at all times when passengers are in the vehicle. |  |  |  |  |  |

| Task Activity : |  | Transporting of Workers |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 1. | Integrety of vehicles | Collisions, break downs, fire | Fatality, injury, property damage production loss. | 5 | 4 | 400 | Random Inspections/ audits on vehicles All vehicles shall be licensed and roadworthy. Pre use inspection sheets to be filled out. All defects shall be immediately reported to the supervisor and defects shall be repaired prior to transporting of workers. All vehicles carrying personnel shall have a fire extinguisher fitted which shall be inspected and fit for use. | 2 | 2 | 16 | Supervisor Transport Driver |  |

| Task Activity : |  | Vehicles entering site: Cranes, Low beds, TLB bakkies, etc |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 1. | Vehicles entering site: Low beds, TLB bakkies, Hi AB. | Driver unaware of site rules | Delays at access gate, time lost. | 2 | 4 | 32 | All contractors to communicate requirements to delivery contractors as well as own transport department on entry requirements and inductions. | 2 | 2 | 16 | Supervisor/ Safety Officer |  |
| 1. | Vehicles entering site: Cranes, Low beds, TLB bakkies, etc | Drivers not familiar with specific site requirements | Entering hazardous areas, Speeding. Damage to property Collisions, | 3 | 4 | 84 | Contractor and Client orientation shall be attended by all drivers. Clear signage shall be posted on site. All drivers shall adhere to the speed limits posted at all times. No unauthorised entry to other work areas. | 2 | 2 | 16 | Safety Officer |  |
| 1. | Vehicles entering site: Cranes, Low beds, TLB bakkies, etc | Drivers/operators under the influence | Injury, damage to property, fatality. | 5 | 4 | 400 | Breathalyzer testing shall be conducted at the entrance gate each morning prior to entering site. Should any persons be suspected of being under the influence on site, testing shall be redone as per procedure. | 2 | 2 | 16 | Security Personnel |  |

| Task Activity : |  | Vehicles entering site: Cranes, Low beds, TLB bakkies, etc |  |  |  |  |  | RA REF : TCBPJV-RA-001 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Task or Activity | Hazards / Risk (Potential danger) (Energy) | Possible Consequence Likely to happen | Initial Risk Score |  |  | Existing & Proposed Control Measures | Revised Risk Score |  |  | Action assigned to | Acceptance of responsibility |
|  |  |  |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  | Consequence (RS) | Likelihood (ROP) | Risk Level |  |  |
| 1. | Vehicles entering site: Cranes, Low beds, TLB bakkies, etc | Drivers/operators Not Competent | Injury, damage to property, fatality. | 5 | 4 | 400 | All operators and drivers shall have the required Unit Standard Training competencies to operate the machinery. Drivers of Construction vehicles shall have a valid drivers licence. | 2 | 2 | 16 | Safety Officer/ Supervisor |  |
|  | Vehicles entering site: Cranes, Low beds, TLB bakkies, etc | Vehicles not compliant | Injury, damage to property, fatality, | 5 | 4 | 400 | Flagman to be posted to control traffic. All Construction vehicles and mobile plant shall be fitted with reverse alarms, amber strobe lights, functioning, as well as all other requirements as per CR 23. | 2 | 2 | 16 | Supervisor |  |

| RISK LEVEL DETERMINATION= ROP X RS X SCF |  | Hazard Effect / Consequence –Risk Severity (RS) of Injury / Incident / Illness, Actual or Potential |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Loss Type | 5- Catastrophic |  | 4- Major | 3 -Moderate | 2- Minor | 1-Insignificant |
| Timeline | May result in overall project timeline overrun of 50% or more |  | May result in overall project timeline overrun of between 20% and less than 50% | May result in overall project timeline overrun of between 5% and less than 20% | May result in overall project timeline overrun of less than 5% | No impact on overall project timeline |
| Budget | May result in overall project budget overrun of 50% or more |  | May result in overall project budget overrun of between 20% and less than 50% | May result in overall project budget overrun of between 5% and less than 20% | May result in overall project budget overrun of less than 5% | No impact on the budget of the project |
| Investment Return – NPV loss | R5b or more |  | R500m to R5b | R50M to less than R500m | R5m to less than R50m | Less than R5m |
| Quality | Significant quality issues that requires sponsorship intervention with significant resource and cost implications for rework |  | Significant quality issues that requires senior project management interaction | Some quality issues that requires immediate management action | Minimal quality issues that can be addressed in a short timeframe with minimal interactions | No impact on quality |
| Safety / Health | Multiple or Single fatality / Impact on health ultimately fatal |  | Loss of quality of life / Irreversible impact on health, Severe accident | Lost time injury or accident / Reversible impact on health | Medical treatment case / Exposure to major health risk, Non LTA | First aid case / Exposure to minor health risk or minor injury |

| Environment |  |  |  |  | EVE1 - Major environmental incident.(Loss of permit, License, Consent or Authorization (Temp or perm) Spills of contaminated liquid > 250 litres |  | EVE2 – Significant environmental incident. Noncompliance with permit / consent (Reportable to Regulatory Authority) Spills > 100 litres, but < 250 litres | EVE3 - Moderate environmental incident, On site pollution incident require remedial action. (Non reportable to Regulatory Authority) |  | EVE4 - Environmental Near miss with the potential to result into EVE 1-3 |  | EVE5 - Poor environmental Acts or conditions, Waste of resources: energy, water etc. Non-conformance to site policy and procedures |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Legal & Regulatory |  |  |  |  | Legal noncompliance with risk of shutdown of operations with significant cost impacts |  | Serious legal concerns and significant impact on operations | Some legal concerns with manageable level of impact |  | Minor legal concerns with minor impact |  | No legal impact |  |  |
| Reputation / Social / Community |  |  |  |  | International impact - international public attention |  | National impact - national public concern | Considerable impact - regional public concern |  | Limited impact - local public concern |  | Slight impact - public awareness may exist but no public concern |  |  |
| Likelihood – Risk Occurrence Probability (ROP) |  |  |  |  | Severity Correction Factor (SCF) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 20 |  | 10 | 7 |  | 4 |  | 1 |  |  |
| 10 Inevitable |  |  | Occurred frequently; has a 90% and higher probability of reoccurring |  | 1000 |  | 400 | 210 |  | 80 |  | 10 |  |  |
| 8 Very High Probability |  |  | Probability of between 60% and less than 90% of occurring |  | 800 |  | 320 | 168 |  | 64 |  | 8 |  |  |
| 6 Probable |  |  | Probability of between 30% and less than 60% of occurring |  | 600 |  | 240 | 126 |  | 48 |  | 6 |  |  |
| 4 Less Probable |  |  | Probability of between 1% and less than 30% of occurring |  | 400 |  | 160 | 84 |  | 32 |  | 4 |  |  |
| 2 Least Probable |  |  | Probability less than 1% of occurring |  | 200 |  | 80 | 42 |  | 16 |  | 2 |  |  |
|  | RISK ASSESSMENT ACKOWLEDGEMENT I, the undersigned, hereby acknowledge that the relevant hazards as identified on the Risk Assessment, was communicated to me and I understand and acknowledge the safety instructions and control measures relevant to my activities. I further more undertake to ensure compliance with the safety regulations and instructions noted. I hereby commit myself to create and maintain a safe work environment. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RA Number & Rev |  |  |  |  |  |  | Facilitator |  |  |  |  |  |  |
|  | No | Name & Surname |  | ID Number |  | Signature |  | No | Name & Surname |  | ID Number |  | Signature |  |
|  | 1 |  |  |  |  |  |  | 16 |  |  |  |  |  |  |

| 2 |  |  |  | 17 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  | 18 |  |  |  |
| 4 |  |  |  | 19 |  |  |  |
| 5 |  |  |  | 20 |  |  |  |
| 6 |  |  |  | 21 |  |  |  |
| 7 |  |  |  | 22 |  |  |  |
| 8 |  |  |  | 23 |  |  |  |
| 9 |  |  |  | 24 |  |  |  |
| 10 |  |  |  | 25 |  |  |  |
| 11 |  |  |  | 26 |  |  |  |
| 12 |  |  |  | 27 |  |  |  |
| 13 |  |  |  | 28 |  |  |  |
| 14 |  |  |  | 29 |  |  |  |
| 15 |  |  |  | 30 |  |  |  |